OSPFv3 & Dual-Stack

Troubleshooting Lab

Multi-Area OSPFv2 / OSPFv3 | IPv4 + IPv6 Dual Stack | DHCP | 10 Embedded Faults

CML Lab ID: 26497480-29c3-466b-bb76-94c06526b86f

Version 1.0   |   April 2026

# 1. Lab Overview

This Cisco Modeling Labs (CML) scenario provides a hands-on troubleshooting environment for multi-area OSPFv2 and OSPFv3 operating over a dual-stacked IPv4/IPv6 network. Students must identify and resolve ten deliberately introduced misconfigurations before full end-to-end connectivity — including dynamic DHCP addressing for three workstations — is achieved.

## 1.1 Learning Objectives

- Diagnose and repair OSPFv3 adjacency failures caused by process-id and timer mismatches

- Identify missing global configuration prerequisites for IPv6 routing

- Resolve dual-stack OSPF area boundary issues for both IPv4 (OSPFv2) and IPv6 (OSPFv3)

- Troubleshoot DHCPv4 pool misconfiguration and DHCPv6 interface binding problems

- Detect and fix a mis-directed DHCP relay helper-address

- Restore IPv4 and IPv6 default-route origination into OSPF for stub segments

- Verify end-to-end connectivity between workstations across all OSPF areas

## 1.2 Topology Summary

The lab uses three OSPF areas with six routers and three workstations:

| Area | Nodes | IPv4 Prefix(es) | IPv6 Prefix(es) |
| --- | --- | --- | --- |
| Area 0 (Backbone) | R1-ABR, R2-ABR | 10.0.0.0/30 | 2001:db8:0::/48 |
| Area 1 | R3-Area1, R4-Area1, WS1 | 10.1.0.0/30, 10.1.1.0/30, 192.168.1.0/24 | 2001:db8:1::/48, 2001:db8:1:100::/64 |
| Area 2 | R5-Area2, R6-Area2, WS2, WS3 | 10.2.0.0/30, 10.2.1.0/30, 192.168.2-3.0/24 | 2001:db8:2::/48, 2001:db8:2:200::/64, 2001:db8:2:300::/64 |

R1 and R2 are Area Border Routers (ABRs) connecting Area 0 to Area 1 and Area 2 respectively. All router-to-router links run both OSPFv2 (IPv4) and OSPFv3 (IPv6) simultaneously.

## 1.3 Physical Topology Diagram

The diagram below illustrates the physical connectivity and area boundaries:

# 2. Lab Requirements

## 2.1 Success Criteria

The lab is considered complete when ALL of the following conditions are met:

- WS1, WS2, and WS3 each receive a valid IPv4 address via DHCP from their respective local pools.

- WS1, WS2, and WS3 each receive a valid IPv6 address via DHCPv6 (stateful) from their respective router.

- WS1 can ping WS2 and WS3 using both IPv4 and IPv6 addresses (cross-area connectivity).

- WS2 can ping WS3 using both IPv4 and IPv6 addresses (intra-area, different gateway).

- All six routers have complete OSPFv2 and OSPFv3 routing tables showing routes for all subnets.

- All OSPFv2 and OSPFv3 adjacencies are in FULL state.

- No OSPF area boundary violations exist (correct area assignments on all interfaces).

## 2.2 Constraints & Rules

- Do NOT restart any router — all fixes must be applied to the running configuration.

- Do NOT use static routes as a substitute for fixing OSPF issues.

- Do NOT modify the physical topology or re-cable any interfaces.

- All DHCP addressing must remain dynamic — do not statically assign addresses to workstations.

- Document every change made using 'show' commands before and after each fix.

## 2.3 Fault Count by Area

| Area | Node(s) | Fault Count |
| --- | --- | --- |
| Area 0 / ABRs | R1-ABR, R2-ABR | 4 |
| Area 1 | R3-Area1, R4-Area1 | 3 |
| Area 2 | R5-Area2, R6-Area2 | 3 |
| TOTAL |  | 10 |

# 3. Addressing Plan

The following table lists all interface IPv4 and IPv6 assignments. Workstation addresses are assigned dynamically via DHCP/DHCPv6 but the pools are shown for reference.

| Node | Interface | IPv4 Address | IPv6 Address | OSPF Area |
| --- | --- | --- | --- | --- |
| R1-ABR | Lo0 | 1.1.1.1/32 | 2001:db8:0:1::1/128 | Area 0 |
| R1-ABR | G0/0 | 10.0.0.1/30 | 2001:db8:0:12::1/64 | Area 0 |
| R1-ABR | G0/1 | 10.1.0.1/30 | 2001:db8:1:13::1/64 | Area 1 |
| R2-ABR | Lo0 | 2.2.2.2/32 | 2001:db8:0:2::2/128 | Area 0 |
| R2-ABR | G0/0 | 10.0.0.2/30 | 2001:db8:0:12::2/64 | Area 0 |
| R2-ABR | G0/1 | 10.2.0.1/30 | 2001:db8:2:25::1/64 | Area 2 |
| R3-Area1 | Lo0 | 3.3.3.3/32 | 2001:db8:1:3::3/128 | Area 1 |
| R3-Area1 | G0/0 | 10.1.0.2/30 | 2001:db8:1:13::2/64 | Area 1 |
| R3-Area1 | G0/1 | 10.1.1.1/30 | 2001:db8:1:34::1/64 | Area 1 |
| R4-Area1 | Lo0 | 4.4.4.4/32 | 2001:db8:1:4::4/128 | Area 1 |
| R4-Area1 | G0/0 | 10.1.1.2/30 | 2001:db8:1:34::2/64 | Area 1 |
| R4-Area1 | G0/1 | 192.168.1.1/24 | 2001:db8:1:100::1/64 | Area 1 LAN |
| R5-Area2 | Lo0 | 5.5.5.5/32 | 2001:db8:2:5::5/128 | Area 2 |
| R5-Area2 | G0/0 | 10.2.0.2/30 | 2001:db8:2:25::2/64 | Area 2 |
| R5-Area2 | G0/1 | 10.2.1.1/30 | 2001:db8:2:56::1/64 | Area 2 |
| R5-Area2 | G0/2 | 192.168.2.1/24 | 2001:db8:2:200::1/64 | Area 2 LAN-A |
| R6-Area2 | Lo0 | 6.6.6.6/32 | 2001:db8:2:6::6/128 | Area 2 |
| R6-Area2 | G0/0 | 10.2.1.2/30 | 2001:db8:2:56::2/64 | Area 2 |
| R6-Area2 | G0/1 | 192.168.3.1/24 | 2001:db8:2:300::1/64 | Area 2 LAN-B |
| WS1-Area1 | eth0 | DHCP (192.168.1.x) | DHCPv6 (2001:db8:1:100::x) | Area 1 |
| WS2-Area2 | eth0 | DHCP (192.168.2.x) | DHCPv6 (2001:db8:2:200::x) | Area 2 |
| WS3-Area2 | eth0 | DHCP (192.168.3.x) | DHCPv6 (2001:db8:2:300::x) | Area 2 |

## 3.1 DHCP Pool Summary

| Pool Name | Served By | IPv4 Network | IPv6 Prefix | Client |
| --- | --- | --- | --- | --- |
| AREA1-WS1 | R3-Area1 | 192.168.1.0/24 | 2001:db8:1:100::/64 | WS1 |
| AREA2-WS2 | R5-Area2 | 192.168.2.0/24 | 2001:db8:2:200::/64 | WS2 |
| AREA2-WS3 | R6-Area2 | 192.168.3.0/24 | 2001:db8:2:300::/64 | WS3 |

# 4. Fault Index (Student Reference)

The following table lists all ten faults by number, node, and title. The Appendix contains complete root-cause analysis and fix commands for each fault. Students should attempt to resolve faults independently before consulting the Appendix.

| # | Node | Fault Title | Protocol / Layer | Area |
| --- | --- | --- | --- | --- |
| 1 | R1-ABR | OSPFv3 Process-ID Mismatch on G0/1 | OSPFv3 | Area 0 / Area 1 boundary |
| 2 | R1-ABR | IPv6 Unicast-Routing Not Enabled | IPv6 | Area 0 |
| 3 | R2-ABR | OSPFv3 Router-ID Not Configured | OSPFv3 | Area 0 |
| 4 | R2-ABR | IPv4 OSPF Area Mismatch on G0/1 (Area 0 vs Area 2) | OSPFv2 (IPv4) | Area 0 / Area 2 boundary |
| 5 | R3-Area1 | DHCPv4 Excluded Range Covers Entire Pool | DHCP (IPv4) | Area 1 |
| 6 | R4-Area1 | LAN Interface Toward WS1 is Administratively Shutdown | Layer 2 / Interface | Area 1 |
| 7 | R4-Area1 | DHCPv6 Server Not Applied and ND Managed-Flag Missing | DHCPv6 | Area 1 |
| 8 | R5-Area2 & R6-Area2 | OSPF Hello/Dead Timer Mismatch Between R5 and R6 | OSPFv2/v3 | Area 2 |
| 9 | R5-Area2 | ip helper-address Points to Non-Existent Server | DHCP Relay (IPv4) | Area 2 |
| 10 | R6-Area2 | Default Route Not Originated into OSPF for WS3 Subnet | OSPFv2/v3 Route Redistribution | Area 2 |

# 5. Recommended Troubleshooting Workflow

## 5.1 Layer-by-Layer Approach

Work from Layer 1 upward. A single shutdown interface can mask multiple apparent OSPF issues.

- Check interface status: show ip interface brief | show ipv6 interface brief

- Verify OSPFv2 neighbors: show ip ospf neighbor

- Verify OSPFv3 neighbors: show ipv6 ospf neighbor

- Check OSPF database completeness: show ip ospf database | show ipv6 ospf database

- Validate routing tables: show ip route ospf | show ipv6 route ospf

- Test DHCP: show ip dhcp binding | show ipv6 dhcp binding

- End-to-end ping test from each workstation

## 5.2 Key Diagnostic Commands

| Command | Purpose |
| --- | --- |
| show ip ospf neighbor detail | Full OSPFv2 neighbor state and timers |
| show ipv6 ospf neighbor detail | Full OSPFv3 neighbor state and timers |
| show ip ospf interface <intf> | OSPFv2 area assignment, hello/dead timers, network type |
| show ipv6 ospf interface <intf> | OSPFv3 area assignment, process-id, timers |
| show ip route ospf | OSPFv2 learned routes in the routing table |
| show ipv6 route ospf | OSPFv3 learned routes in the routing table |
| show ip dhcp binding | Active DHCPv4 address assignments |
| show ipv6 dhcp binding | Active DHCPv6 address assignments |
| show ipv6 dhcp interface <intf> | DHCPv6 server/relay status per interface |
| debug ip dhcp server events | Real-time DHCP discover/offer/request/ack |
| show running-config ¦ section ospf | Full OSPF configuration snapshot |
| show ipv6 ospf database summary | OSPFv3 LSA database completeness check |

APPENDIX: Fault Solutions & Fix Commands

This appendix contains the complete root-cause analysis, verification steps, and exact IOS commands required to resolve each of the ten faults embedded in this lab. Each fault section includes the symptom observed, the configuration error, and the precise fix.

INSTRUCTOR NOTE: This section should be withheld from students during the initial troubleshooting phase.

Fault 1: OSPFv3 Process-ID Mismatch on G0/1

| Node | R1-ABR |
| --- | --- |
| Protocol/Layer | OSPFv3 |
| OSPF Area | Area 0 / Area 1 boundary |
| Symptom | OSPFv3 adjacency never forms between R1 and R3 across the Area 0/1 boundary. 'show ipv6 ospf neighbor' shows no entry for R3. |
| Root Cause | R1's G0/1 interface is assigned to OSPFv3 process 2 instead of process 1. Since R3 runs process 1, the hello packets are silently discarded. |
| Verification | show ipv6 ospf neighbor show ipv6 ospf interface GigabitEthernet0/1 |
| Fix Commands | interface GigabitEthernet0/1 no ipv6 ospf 2 area 1 ipv6 ospf 1 area 1 |

Fault 2: IPv6 Unicast-Routing Not Enabled

| Node | R1-ABR |
| --- | --- |
| Protocol/Layer | IPv6 |
| OSPF Area | Area 0 |
| Symptom | R1 does not forward any IPv6 packets. Pings to/from IPv6 addresses through R1 fail. OSPFv3 hellos may send but routes are not installed. |
| Root Cause | The global command 'ipv6 unicast-routing' is missing on R1. Without this, the router will not forward IPv6 traffic regardless of any interface configuration. |
| Verification | show running-config ¦ include ipv6 unicast-routing show ipv6 route |
| Fix Commands | ipv6 unicast-routing |

Fault 3: OSPFv3 Router-ID Not Configured

| Node | R2-ABR |
| --- | --- |
| Protocol/Layer | OSPFv3 |
| OSPF Area | Area 0 |
| Symptom | OSPFv3 process on R2 may fail to start or use an unstable auto-selected router-id. Adjacencies with R1 and R5 flap or do not form on the IPv6 plane. |
| Root Cause | The 'router-id' statement is missing under 'ipv6 router ospf 1'. OSPFv3 requires an explicit 32-bit router-id since it does not rely on IPv4 addresses for this purpose. |
| Verification | show ipv6 ospf ¦ include Router ID show ipv6 ospf neighbor |
| Fix Commands | ipv6 router ospf 1 router-id 2.2.2.2 |

Fault 4: IPv4 OSPF Area Mismatch on G0/1 (Area 0 vs Area 2)

| Node | R2-ABR |
| --- | --- |
| Protocol/Layer | OSPFv2 (IPv4) |
| OSPF Area | Area 0 / Area 2 boundary |
| Symptom | IPv4 routes for the 10.2.0.0/30 and 192.168.2-3.0/24 subnets are missing from R1 and R3/R4. IPv4 connectivity from Area 1 to Area 2 workstations fails. |
| Root Cause | R2's GigabitEthernet0/1 (link toward R5/Area2) is configured 'ip ospf 1 area 0' instead of 'area 2'. This incorrectly places a non-backbone interface into Area 0, violating the area boundary and preventing proper LSA flooding into Area 2. |
| Verification | show ip ospf interface GigabitEthernet0/1 show ip route ospf |
| Fix Commands | interface GigabitEthernet0/1 no ip ospf 1 area 0 ip ospf 1 area 2 |

Fault 5: DHCPv4 Excluded Range Covers Entire Pool

| Node | R3-Area1 |
| --- | --- |
| Protocol/Layer | DHCP (IPv4) |
| OSPF Area | Area 1 |
| Symptom | WS1 cannot obtain an IPv4 address via DHCP. 'show ip dhcp binding' on R3 shows no active bindings. WS1 falls back to APIPA (169.254.x.x). |
| Root Cause | The 'ip dhcp excluded-address' command covers 192.168.1.1 through 192.168.1.254 — the entire subnet. No addresses remain in the pool to be offered. |
| Verification | show ip dhcp pool show ip dhcp binding show ip dhcp conflict |
| Fix Commands | no ip dhcp excluded-address 192.168.1.1 192.168.1.254 ip dhcp excluded-address 192.168.1.1 192.168.1.10 |

Fault 6: LAN Interface Toward WS1 is Administratively Shutdown

| Node | R4-Area1 |
| --- | --- |
| Protocol/Layer | Layer 2 / Interface |
| OSPF Area | Area 1 |
| Symptom | WS1 has no Layer 2 or Layer 3 connectivity at all. The GigabitEthernet0/1 interface on R4 shows 'administratively down' in 'show interface' output. |
| Root Cause | The 'shutdown' command was left applied to R4's G0/1 interface (the WS1 LAN port). The interface must be enabled with 'no shutdown'. |
| Verification | show interface GigabitEthernet0/1 show ip interface brief |
| Fix Commands | interface GigabitEthernet0/1 no shutdown |

Fault 7: DHCPv6 Server Not Applied and ND Managed-Flag Missing

| Node | R4-Area1 |
| --- | --- |
| Protocol/Layer | DHCPv6 |
| OSPF Area | Area 1 |
| Symptom | WS1 cannot obtain an IPv6 address via DHCPv6. Stateless autoconfiguration (SLAAC) addresses appear but no managed address is assigned. 'show ipv6 dhcp binding' shows no WS1 entry. |
| Root Cause | Two omissions on R4's G0/1: (1) 'ipv6 dhcp server AREA1-V6-WS1' is not applied to the interface, so the pool on R3 is unreachable for stateful assignments; (2) 'ipv6 nd managed-config-flag' is missing, so clients do not attempt DHCPv6 at all. |
| Verification | show ipv6 dhcp binding show ipv6 dhcp interface GigabitEthernet0/1 show ipv6 interface GigabitEthernet0/1 |
| Fix Commands | interface GigabitEthernet0/1 ipv6 dhcp server AREA1-V6-WS1 ipv6 nd managed-config-flag ipv6 nd other-config-flag |

Fault 8: OSPF Hello/Dead Timer Mismatch Between R5 and R6

| Node | R5-Area2 & R6-Area2 |
| --- | --- |
| Protocol/Layer | OSPFv2/v3 |
| OSPF Area | Area 2 |
| Symptom | The OSPFv2 and OSPFv3 adjacency between R5 and R6 never reaches FULL state. 'show ip ospf neighbor' and 'show ipv6 ospf neighbor' show no entry or flapping state on the 10.2.1.0/30 link. |
| Root Cause | R5's G0/1 is configured with hello-interval 10 / dead-interval 40, while R6's G0/0 uses hello-interval 30 / dead-interval 120. OSPF requires matching timers on both sides of a link. |
| Verification | show ip ospf interface GigabitEthernet0/1 (on R5) show ip ospf interface GigabitEthernet0/0 (on R6) show ip ospf neighbor |
| Fix Commands | ! On R5 G0/1 AND R6 G0/0 — set both to the same values: interface GigabitEthernet0/1 ! R5 no ip ospf hello-interval 10 no ip ospf dead-interval 40 no ipv6 ospf hello-interval 10 no ipv6 ospf dead-interval 40 interface GigabitEthernet0/0 ! R6 no ip ospf hello-interval 30 no ip ospf dead-interval 120 no ipv6 ospf hello-interval 30 no ipv6 ospf dead-interval 120 |

Fault 9: ip helper-address Points to Non-Existent Server

| Node | R5-Area2 |
| --- | --- |
| Protocol/Layer | DHCP Relay (IPv4) |
| OSPF Area | Area 2 |
| Symptom | WS2 cannot obtain an IPv4 DHCP address via relay. Broadcasts from WS2 are forwarded to 192.168.3.99 which does not exist. The local pool on R5 is not used because helper-address overrides it. |
| Root Cause | R5's G0/2 (WS2 LAN) has 'ip helper-address 192.168.3.99' which points to an invalid host. The DHCP pool AREA2-WS2 is defined locally on R5; either remove the helper-address (to serve locally) or fix it to point to the correct server. |
| Verification | show running-config interface GigabitEthernet0/2 show ip dhcp binding debug ip dhcp server events |
| Fix Commands | interface GigabitEthernet0/2 no ip helper-address 192.168.3.99 |

Fault 10: Default Route Not Originated into OSPF for WS3 Subnet

| Node | R6-Area2 |
| --- | --- |
| Protocol/Layer | OSPFv2/v3 Route Redistribution |
| OSPF Area | Area 2 |
| Symptom | WS3 (192.168.3.0/24 and 2001:db8:2:300::/64) can reach R6 but cannot reach any host outside Area 2. 'show ip route' on WS3's gateway R6 shows no default route being advertised to other areas. |
| Root Cause | 'default-information originate always' is missing from both 'router ospf 1' and 'ipv6 router ospf 1' on R6. As a result, no default or summary route is propagated toward the backbone, isolating WS3's subnet from inter-area and inter-domain traffic. |
| Verification | show ip ospf database external show ip route ¦ include 0.0.0.0 show ipv6 route ¦ include ::/0 |
| Fix Commands | router ospf 1 default-information originate always ! ipv6 router ospf 1 default-information originate always |

# Appendix B: Fully Corrected Configuration Snippets

After resolving all ten faults, the following summarises the correct state for the key configuration items:

### R1-ABR — Corrected G0/1 and global IPv6

ipv6 unicast-routing                          ! Fix #2

!

interface GigabitEthernet0/1

no ipv6 ospf 2 area 1                        ! Fix #1

ipv6 ospf 1 area 1

### R2-ABR — Corrected router-id and area assignment

ipv6 router ospf 1

router-id 2.2.2.2                            ! Fix #3

!

interface GigabitEthernet0/1

no ip ospf 1 area 0                          ! Fix #4

ip ospf 1 area 2

### R3-Area1 — Corrected DHCP excluded range

no ip dhcp excluded-address 192.168.1.1 192.168.1.254   ! Fix #5

ip dhcp excluded-address 192.168.1.1 192.168.1.10

### R4-Area1 — Enable interface and DHCPv6

interface GigabitEthernet0/1

no shutdown                                  ! Fix #6

ipv6 dhcp server AREA1-V6-WS1               ! Fix #7

ipv6 nd managed-config-flag

ipv6 nd other-config-flag

### R5-Area2 — Fix timers and helper-address

interface GigabitEthernet0/1

no ip ospf hello-interval 10                 ! Fix #8

no ip ospf dead-interval 40

no ipv6 ospf hello-interval 10

no ipv6 ospf dead-interval 40

!

interface GigabitEthernet0/2

no ip helper-address 192.168.3.99            ! Fix #9

### R6-Area2 — Fix timers and default-information originate

interface GigabitEthernet0/0

no ip ospf hello-interval 30                 ! Fix #8

no ip ospf dead-interval 120

no ipv6 ospf hello-interval 30

no ipv6 ospf dead-interval 120

!

router ospf 1

default-information originate always         ! Fix #10

!

ipv6 router ospf 1

default-information originate always